### ‘ME AND WILLY BAND’ Tech Rider 
CONTACT: Pat Winfield: 216-410-5297/ me@meandwilly.com

band members & general description

Willy (dobro, harmonica, electric fiddle); Pat (acoustic guitar); Bruce (upright bass); Raven (acoustic guitar); Brad (mandolin) (occasionally a banjo/fiddler added).  All members sing both lead and harmony vocals.

Me and Willy play ‘from blues to bluegrass’ and are an acoustically oriented, roots band, preferring a crisp sound and don’t necessarily need to be really loud.  Each person uses a separate mic and boom stand for singing; mandolin requires a uni-directional mic; Willy’s instruments and Bruce’s bass are run through separate amps and with a direct XLR out; the acoustic guitars are run directly (Pat’s has no separate control with a hot Martin bridge pickup; she also requires a hot vocal mic)

| Chan# | Input Description |
| --- | --- |
| 1 XLR | Willy’s vocal |
| 2 XLR | Pat’s vocal(hot) |
| 3 XLR | Bruce’s vocal |
| 4 XLR | Raven’s vocal |
| 5 XLR | Brad’s vocal |
| 6 XLR | Brad’s instr mic |
| 7 XLR | Willy’s instr amp |
| 8 HiZ | Pat’s guitar |
| 9 HiZ | Bass amp |
| 10HiZ | Raven’s guitar |
| 11 | xtra opt inst/mic |
| 12 | xtra opt inst/mic |

Fender Acousti-sonic Jr. XLR  out (Willy’s)

Double Bass Amp XLR out (Bruce)

A/C

table (if possible) or chair for hats, harps, rosin & stuff

double  bass/stand

mic 3 Bruce

guitar/ stand

mic 6 (uni)

fiddle

mic 2

Pat

mic 4

Raven

mic 1

Willy

mic 5

Brad

mandolin/

stand

guitar] stand

dobro 2

dobro 1

monitor

monitor

STAGE PLOT: performance area is flexible but preferably min of 10’ by 6’/60 sq ft

LIGHTING
- No specific lighting requirements.

STAGE MONITOR REQUIREMENTS
- We will need a minimum of 2 floor monitors on stage on a single monitor mix

MICS AND PROCESSING
- 5 high quality vocal mics (all band members sing lead and backup) with boom stands, Willy and Pat generally use EV NB767 but SM58s or equivalent are fine. 
- we don't use any processing 
- (please contact us before the show if we should need to bring our own mics)

MAIN SPEAKERS
- all we need is clear vocal reproduction over the normal voice singing range.

ACCESSORIES
- 2 XLR cords, 6 HiZ cords, table (min. 2’x3’) or chair on stage to hold items.

OTHER
- 2-3 bar height stools for Willy, Pat and Bruce are nice but optional for 1.25 hours or less.  
- we would never say, “Hey, why are there 5 cold bottles of natural spring water here?” or after the  show (and in appropriate settings), “Who the heck delivered  this Guinness draught and Great Lakes Dortmunder?”

Version 060617 This Tech Rider supersedes all lower numbered versions.